Faculty of Technology

University of Sri Jayewardenepura

Assignment

Emerging Technologies

Name : K.P.V.Perera

Index Number: ICT/21/897

Table of Content

1.	INTRODUCTION AND PROBLEM STATEMENT	3

1.1 Background and Motivation	3

1.2 Problem Definition	3

1.3 Why Fuzzy Logic?	4

1.3.1 Limitations of Crisp Logic	4

1.3.2 Advantages Over PID Controllers	5

1.3.3 Real-World Applicability	7

1.4 Project Objectives	8

1.4.1 Primary Objectives	8

1.4.2 Secondary Objectives	9

2.	SYSTEM MODELING AND DESIGN JUSTIFICATION	9

2.1.1 Tomato (Solanum lycopersicum)	9

3.	MEMBERSHIP FUNCTIONS & RULES	12

Humidity (30-100%) - 5 Fuzzy Sets	12

4.	IMPLEMENTATION & ADAPTATION	14

4.3.2 Sugeno Controller	16

4.4 Dynamic Adaptation	16

4.4.1 Adaptation Mechanism	16

5.	PERFORMANCE ANALYSIS	18

6.	OPTIMIZATION & RESULTS	20

7.	CONCLUSION & FUTURE WORK	23

7.1 Overall System Performance	23

7.2 Key Findings	24

7.3 Practical Implications	24

8. GUI interface for live simulation	25

# INTRODUCTION AND PROBLEM STATEMENT

## 1.1 Background and Motivation

The global agricultural sector faces unprecedented pressure to increase food production while minimizing environmental impact and resource consumption. Controlled environment agriculture (CEA), particularly greenhouse cultivation, has emerged as a promising solution, offering the potential to optimize growing conditions year-round while protecting crops from weather extremes and pests.

However, the success of greenhouse operations fundamentally depends on maintaining precise environmental control. Temperature and humidity must be carefully managed to match each plant species' requirements, which themselves vary across different growth stages. This presents a complex, multi-variable control problem that traditional automation approaches struggle to address effectively.

Consider a real-world greenhouse scenario: a grower cultivates tomatoes during spring and summer, switches to lettuce for fall production, and maintains orchids year-round in a separate section. Each crop demands different climate conditions. Tomatoes thrive in warm, moderately humid environments during vegetative growth but require cooler conditions during flowering to set fruit properly. Lettuce prefers consistently cool temperatures throughout its lifecycle and will "bolt" (prematurely flower and become bitter) if exposed to heat. Orchids, being tropical epiphytes, need warmth and high humidity that would devastate lettuce.

## 1.2 Problem Definition

The Core Challenge:

Design and implement an intelligent, adaptive control system capable of maintaining optimal temperature and humidity in a greenhouse cultivating three distinct plant species across multiple growth stages, while minimizing energy consumption and providing smooth, plant-friendly control transitions.

System Specifications:

Environmental Inputs:

- Temperature: Measured range 5-40°C (sensors accurate to ±0.5°C)

- Humidity: Measured range 30-100% RH (sensors accurate to ±2%)

- Growth Stage: Discrete values representing Seedling (0), Vegetative (1), or Flowering (2)

Control Outputs:

- Heater/Cooling Power: Continuous range 0-100% controlling HVAC system

- Misting System: Continuous range 0-100% controlling humidification intensity

Target Plant Species:

- Tomato (Solanum lycopersicum) - Warm-season vegetable crop

  - Commercial importance: $2+ billion annual US production

  - Temperature sensitivity: Requires different conditions per growth stage

  - Represents mid-range climate preferences

- Lettuce (Lactuca sativa) - Cool-season leafy green

  - Commercial importance: Year-round demand for fresh greens

  - Temperature sensitivity: Bolting risk if conditions too warm

  - Represents lower temperature range

- Orchid (Orchidaceae family) - Tropical ornamental plant

  - Commercial importance: High-value specialty crop ($200M+ US market)

  - Climate sensitivity: Requires warm, humid conditions

  - Represents upper temperature and humidity range

Performance Requirements:

- Response Time: Control decisions within 100ms to enable real-time operation

- Accuracy: Maintain conditions within ±2°C of target temperature and ±5% of target humidity

- Stability: Minimize oscillations and provide smooth control transitions

- Adaptability: Switch between plant types without manual re-tuning

- Energy Efficiency: Optimize actuator usage while meeting comfort requirements

Implementation Constraints:

- Must implement both Mamdani and Takagi-Sugeno fuzzy inference systems

- Minimum 25 fuzzy rules per controller (50+ implemented)

- Must demonstrate a dynamic adaptation mechanism

- Must conduct a minimum of 20 test simulations for performance evaluation

- Must apply optimization techniques to improve system parameters

## 1.3 Why Fuzzy Logic?

The selection of fuzzy logic as the control methodology stems from careful analysis of greenhouse control requirements versus available control technologies.

### 1.3.1 Limitations of Crisp Logic

Traditional binary logic operates on absolute categories: temperature is either "hot" or "not hot," humidity is either "high" or "not high." This Boolean approach fails to capture the gradual nature of real-world phenomena. Let’s look at a practical example.

1.3.1.1 Practical Example:

Consider a tomato plant in vegetative growth with an optimal range of 22-28°C:

1. If we use the Binary Logic Approach:

IF temperature > 25°C THEN heating = OFF

IF temperature ≤ 25°C THEN heating = ON

At 25.1°C, the heater shuts off completely. At 24.9°C, it runs at maximum power. A 0.2°C measurement fluctuation (well within sensor accuracy) causes full actuation changes.

Result: The system oscillates wildly, wasting energy and stressing plants.

2. If we use the fuzzy Logic Approach

IF temperature is slightly_warm THEN heating is low

IF temperature is optimal THEN heating is very_low

IF temperature is warm THEN heating is off

Benefit: At 25°C, temperature might be "60% optimal and 40% slightly_warm," producing smooth, proportional control that adjusts gradually as conditions change.

### 1.3.2 Advantages Over PID Controllers

PID (Proportional-Integral-Derivative) controllers have served automation well for decades, but greenhouse applications expose fundamental limitations:

1.3.2.1 Mathematical Complexity:

PID controllers calculate output as:

u(t) = Kp·e(t) + Ki·∫e(τ)dτ + Kd·de(t)/dt

Where:

- Kp, Ki, Kd are tuning parameters

- e(t) is the error between setpoint and measurement

- The integral term accounts for accumulated error

- The derivative term predicts future error

The Problems:

- Tuning Difficulty: Finding optimal Kp, Ki, Kd values requires expertise and trial-and-error. Each plant-stage combination needs different parameters—that's 9 parameter sets for our three plants and three stages.

- Non-Linear Systems: PIDs assume linear system response. Greenhouses are inherently non-linear:

  - Misting cools through evaporation (temperature-humidity coupling)

  - Heating reduces relative humidity even without ventilation

  - Thermal mass causes asymmetric heating and cooling rates

  - Plant transpiration varies with growth stage

- Multiple Inputs: Standard PID handles one input and one output. Greenhouse control needs to consider temperature AND humidity AND growth stage simultaneously to make smart decisions.

- Adaptation: Switching from tomato (22-28°C optimal) to lettuce (15-22°C optimal) requires recalculating all PID parameters. There's no straightforward way to adapt.

Fuzzy Logic Solutions:

| Aspect | PID Controller | Fuzzy Logic Controller |
| --- | --- | --- |
| Mathematical Model | Requires precise transfer function | Works with linguistic descriptions |
| Tuning Process | Complex, requires control expertise | Intuitive, uses if-then rules |
| Non-Linear Handling | Poor—assumes linearity | Excellent—inherently non-linear |
| Multiple Inputs | Difficult—MIMO requires advanced techniques | Natural—easily handles multiple inputs |
| Interpretability | Obscure—parameters lack physical meaning | Clear—rules mirror human reasoning |
| Adaptation | Requires complete re-tuning | Simple—update target ranges |
| Coupled Variables | Cannot handle directly | Manages coupled effects naturally |
| Expert Knowledge | Hard to incorporate | Direct translation of expertise |

### 1.3.3 Real-World Applicability

A greenhouse operator knows: "When it's getting quite warm and the humidity is high, I ease off the heating completely and reduce misting to prevent condensation that causes fungal disease."

PID translation: Unclear how do you encode "quite warm," "high humidity," and "ease off" into Kp, Ki, Kd values?

Fuzzy translation:

IF temperature is quite_warm AND humidity is high

THEN heater is off AND misting is low

Direct, interpretable, maintainable.

1.3.4 Fuzzy Logic's Unique Strengths

- Handling Uncertainty:

Sensor readings include noise. Biological systems have individual variation. Fuzzy logic embraces uncertainty rather than fighting it:

- Temperature sensor reads 23.4°C ± 0.5°C

- Fuzzy system treats this as "mostly optimal, slightly warm"

- Control response proportional to degree of membership

- No false precision, no brittle thresholds

2.  Linguistic Variables:

Fuzzy logic allows us to work with terms humans understand:

- "Very cold," "cold," "optimal," "warm," "very warm"

- "Very low," "low," "optimal," "high," "very high"

- "Seedling," "vegetative," "flowering"

This makes system behavior transparent and debuggable

3.  Smooth Control:

Membership functions overlap, ensuring smooth transitions:

- At 23°C: Temperature is "70% optimal and 30% slightly_warm"

- At 24°C: Temperature is "50% optimal and 50% slightly_warm"

- At 25°C: Temperature is "30% optimal and 70% slightly_warm"

- Control output changes gradually, not abruptly

## 1.4 Project Objectives

This project demonstrates fuzzy logic's practical effectiveness through comprehensive implementation and evaluation:

### 1.4.1 Primary Objectives

1.4.1. 1. Comprehensive Fuzzy System Design

- Design 5+ membership functions per input variable (temperature, humidity, growth stage)

- Design 5+ membership functions per output variable (heater power, misting intensity)

- Justify all design choices based on plant physiology and control requirements

- Ensure complete coverage of operating ranges

1.4.1.2. Dual Controller Implementation

- Implement Mamdani fuzzy inference system emphasizing interpretability

- Implement Takagi-Sugeno fuzzy inference system for computational efficiency

- Develop minimum 25 rules per controller (50+ actually implemented)

- Compare and contrast both approaches

1.4.1.3. Dynamic Adaptation Mechanism

- Create automatic plant type switching capability

- Implement growth stage transition handling

- Demonstrate smooth adaptation without control interruption

- Validate adaptation across all plant-stage combinations

1.4.1.4. Rigorous Performance Evaluation

- Generate 20+ randomized test scenarios

- Execute simulations for both controllers

- Measure and compare:

  - Response time (computational efficiency)

  - Control error (accuracy)

  - Energy usage (efficiency)

  - Output smoothness (stability)

- Determine which controller performs better and why

1.4.1.5. Parameter Optimization

- Implement Genetic Algorithm for membership function parameter tuning

- Implement Particle Swarm Optimization as alternative technique

- Compare optimization approaches

- Quantify performance improvement

### 1.4.2 Secondary Objectives

1.4.2.1 Interactive Visualization

- Develop GUI interface for real-time interaction

- Provide live plotting of control outputs

- Enable manual parameter adjustment

- Support both controller types

1.4.2.2 Comprehensive Documentation

- Generate detailed technical report

- Create visualizations of all membership functions

- Produce performance comparison charts

- Document code with clear comments

1.4.2.3 Analysis and Critical Thinking

- Identify system limitations

- Discuss real-world implementation challenges

- Propose future improvements

- Connect theory to practice

# SYSTEM MODELING AND DESIGN JUSTIFICATION

2.1 Plant Species Selection

Three commercially important plant species were selected to represent diverse climate requirements across the greenhouse temperature and humidity spectrum.

### 2.1.1 Tomato (Solanum lycopersicum)

Tomatoes represent warm-season crops with moderate climate needs, serving as the baseline reference for system design.

Climate Requirements:-

| Growth Stage | Temperature | Humidity | Rationale |
| --- | --- | --- | --- |
| Seedling | 20-25°C | 65-75% | Warmth promotes germination |
| Vegetative | 22-28°C | 60-70% | Active growth tolerates heat |

Selection Justification:- Tomatoes have well-documented requirements, significant commercial value, and represent mid-range conditions suitable for testing balanced control strategies.

2.1.2 Lettuce (Lactuca sativa)

Lettuce represents cool-season crops, testing the system's lower temperature range capabilities.

Climate Requirements:

| Growth Stage | Temperature | Humidity | Rationale |
| --- | --- | --- | --- |
| Seedling | 15-20°C | 70-80% | Cool germination prevents bolting |
| Vegetative | 15-22°C | 65-75% | Maintains crisp texture |

Selection Justification: Lettuce provides contrast to tomato, requiring consistently cooler conditions and demonstrating system adaptability to low-temperature crops.

2.1.3 Orchid (Orchidaceae family)

Orchids represent tropical plants requiring warm, humid conditions, testing the system's upper range.

Climate Requirements:

| Growth Stage | Temperature | Humidity | Rationale |
| --- | --- | --- | --- |
| Seedling | 22-28°C | 75-85% | Tropical warmth for young plants |
| Vegetative | 24-30°C | 70-80% | High humidity mimics native habitat |

Selection Justification: Orchids test maximum temperature and humidity capabilities while representing high-value specialty crops.

2.2 Climate Requirements Analysis

2.2.1 Comprehensive Requirements Table

2.2.2 Key Observations

2.2.2.1 Temperature Analysis:

- Range Span: 12°C (lettuce flowering) to 30°C (orchid vegetative) = 18°C total operating range

- Coolest Requirements: Lettuce consistently 5-8°C cooler than other species

- Warmest Requirements: Orchid vegetative stage at 24-30°C

- Common Pattern: All species prefer warmer conditions during seedling, peak warmth during vegetative growth, and moderate cooling during flowering

2.2.2.2 Humidity Analysis:

- Range Span: 50% (tomato flowering) to 85% (orchid seedling) = 35% total range

- Driest Conditions: Tomato flowering at 50-65%

- Moistest Conditions: Orchid seedling at 75-85%

- Common Pattern: Seedlings universally need higher humidity; flowering stages require lower humidity to prevent disease

2.2.2.3 Control Implications:

- System must handle wide operating ranges (18°C temperature span, 35% humidity span)

- Smooth transitions needed between growth stages

- Species-specific adaptation is critical

- Growth stage has significant impact on optimal conditions

# MEMBERSHIP FUNCTIONS & RULES

3.1 Input Variables

Temperature (5-40°C) - 5 Fuzzy Sets

| Set | Type | Parameters | Meaning |
| --- | --- | --- | --- |
| Very Cold | Trap | [5,5,12,16] | Needs max heating |
| Cold | Tri | [12,18,22] | Below optimal |

### Humidity (30-100%) - 5 Fuzzy Sets

| Set | Type | Parameters | Meaning |
| --- | --- | --- | --- |
| Very Low | Trap | [30,30,40,50] | Max misting |
| Low | Tri | [40,55,65] | Increase misting |
| Optimal | Tri | [60,70,80] | Ideal moisture |
| High | Tri | [75,85,92] | Reduce misting |
| Very High | Trap | [88,95,100,100] | Disease risk |

Growth Stage (0-2) - 3 Fuzzy Sets

| Set | Parameters | Represents |
| --- | --- | --- |
| Seedling | [0,0,1] | Young plants |
| Vegetative | [0.5,1,1.5] | Active growth |
| Flowering | [1,2,2] | Reproductive |

3.2 Output Variables

Both outputs use same structure (0-100%):

| 3.2 Output Variables | 3.2 Output Variables | 3.2 Output Variables |
| --- | --- | --- |
| 3.2 Output Variables | 3.2 Output Variables | 3.2 Output Variables |
| 3.2 Output Variables | 3.2 Output Variables | 3.2 Output Variables |
| 3.2 Output Variables | 3.2 Output Variables | 3.2 Output Variables |
| 3.2 Output Variables | 3.2 Output Variables | 3.2 Output Variables |
| 3.2 Output Variables | 3.2 Output Variables | 3.2 Output Variables |

3.3 Fuzzy Rules

Total: 50 rules (25 temperature + 25 humidity)

Sample Temperature Rules:

R1: IF temp is very_cold AND stage is seedling → heater is maximum

R2: IF temp is cold AND stage is vegetative → heater is medium

…..

R11: IF temp is optimal AND humidity is optimal → heater is off

R21: IF temp is very_warm → heater is off

Sample Humidity Rules:

R26: IF humidity is very_low AND temp is warm → misting is maximum

R30: IF humidity is optimal AND temp is optimal → misting is off

…..

R44: IF humidity is high AND temp is cold → misting is off

R50: IF humidity is very_high → misting is off

3.4 Defuzzification

Mamdani: Centroid method - Intuitive, interpretable
Sugeno: Weighted average - Fast, efficient

# IMPLEMENTATION & ADAPTATION

4.1 Development Environment

Programming Language: Python 3.14
Key Libraries:

- scikit-fuzzy - Fuzzy logic operations

- NumPy - Numerical computations

- Matplotlib - Visualizations

- Pandas - Data handling

4.2 System Architecture

1. FuzzyGreenhouseController

- Creates membership functions

- Defines 50+ fuzzy rules

- Builds Mamdani/Sugeno controllers

2. AdaptiveFuzzyController

- Wraps base controller

- Handles plant type switching

- Manages growth stage transitions

3. PerformanceEvaluator

- Runs test simulations

- Collects performance metrics

- Generates comparison data

4.3 Controller Implementation

4.3.1  Mamdani Controller

heater_power = ctrl.Consequent(np.arange(0, 101, 1), 'heater_power')

misting = ctrl.Consequent(np.arange(0, 101, 1), 'misting')

Characteristics:

- Uses linguistic output sets

- Centroid defuzzification

- Better interpretability

### 4.3.2 Sugeno Controller

heater_power = ctrl.Consequent(np.arange(0, 101, 1), 'heater_power',

defuzzify_method='som')

Characteristics:

- Linear output functions

- Sum of Products defuzzification

- Faster computation

## 4.4 Dynamic Adaptation

### 4.4.1 Adaptation Mechanism

When plant type changes:

def adapt_to_plant(new_plant_type):

1. Load new plant requirements

2. Update target temperature range

3. Update target humidity range

4. Rebuild controller with same rules

5. Apply smooth transition

Key Feature: Rules stay constant, only targets change!

4.4.2 Adaptation Results

Test Scenario: Temp=22°C, Humidity=65%, Stage=Seedling

Conclusion:- System successfully adapts control outputs based on species requirements without manual re-tuning.

4.5 Code Highlights

- Key Function: Compute Control

def compute(self, temp, humid, stage):

"""Calculate control outputs"""

self.controller.input['temperature'] = temp

self.controller.input['humidity'] = humid

self.controller.input['growth_stage'] = stage

self.controller.compute()

return (

self.controller.output['heater_power'],

self.controller.output['misting']

)

- Adaptation with Error Correction

def compute_with_adaptation(self, temp, humid, stage):

"""Compute with adaptive fine-tuning"""

# Get base controller output

heater, misting = self.base_controller.compute(temp, humid, stage)

# Calculate error from target

target_temp, target_humid = self.get_target_ranges(stage)

temp_error = temp - (target_temp[0] + target_temp[1]) / 2

# Apply adaptive correction

if temp_error > 5:  # Too hot

heater = max(0, heater - 20)

elif temp_error < -5:  # Too cold

heater = min(100, heater + 20)

return heater, misting

# PERFORMANCE ANALYSIS

5.1 Test Methodology

Test Cases: 20 random scenarios
Temperature Range: 10-35°C
Humidity Range: 35-95%
Growth Stages: All three tested

Metrics Measured:

- Response time (ms)

- Average error (°C/%)

- Energy usage (%)

- Smoothness score

5.2 Results

Performance Comparison Table

Visual Analysis

5.3 Winner Analysis

Overall Winner: SUGENO

# OPTIMIZATION & RESULTS

6.1 Optimization Objectives

Goal: Improve fuzzy membership function parameters to minimize control error.

Target Parameters:

- Temperature membership function centers

- Humidity membership function centers

- Total: 6 parameters optimized

6.2 Genetic Algorithm

Parameters:

- Population: 50

- Generations: 100

- Mutation Rate: 0.1

- Crossover Rate: 0.8

Process:

- Initialize random population

- Evaluate fitness (inverse of error)

- Select best performers (tournament)

- Crossover and mutation

- Repeat until convergence

Results:

- Converged at generation ~40

- Final fitness: 1.0000

- Improvement: 18,078.5%

6.3 Particle Swarm Optimization

Parameters:

- Particles: 30

- Iterations: 100

- Inertia: 0.7

- Cognitive/Social: 1.5

Process:

- Initialize particle swarm

- Evaluate fitness

- Update velocities and positions

- Track global best

- Converge to optimum

Results:

- Converged at iteration ~20 (faster than GA!)

- Final fitness: 1.0000

- Improvement: 18,078.5%

6.4 Optimization Comparison

Results Table

Optimization Results

| Method | Fitness | Improvement (%) |
| --- | --- | --- |
| Original | 0.0055 | 0% |
| Genetic Algorithm | 1.0000 | 18,078.5% |
| Particle Swarm | 1.0000 | 18,078.5% |

Parameter Changes

6.5 Key Findings

Winner: PSO - Faster convergence (20 vs 40 iterations)

Improvements:

- 18,078% performance boost

- Near-perfect fitness achieved

- Both methods successful

- PSO more efficient

Impact:

- Dramatically reduced control error

- Better tracking of target conditions

- Smoother actuator operation

- Validated optimization approach

# CONCLUSION & FUTURE WORK

## 7.1 Overall System Performance

Key Achievements:

- Successfully controlled three plant species

- Maintained conditions within ±2°C and ±5% humidity

- Dynamic adaptation working seamlessly

- 18,078% improvement through optimization

- Both controllers performed well

Winner : SUGENO

## 7.2 Key Findings

- Fuzzy logic is superior to traditional PID for multi-species greenhouses

- Adaptation mechanism enables zero-downtime plant switching

- Optimization is critical - manual tuning insufficient

- PSO converges faster than GA (20 vs 40 iterations)

- Both inference types work - choice depends on priority (interpretability vs speed)

## 7.3 Practical Implications

For Commercial Greenhouses:

- Reduced energy costs through optimized control

- Better plant health from stable conditions

- Easy crop rotation without re-programming

- Scalable to multiple zones

For Research:

- Validates fuzzy logic for agricultural automation

- Demonstrates optimization necessity

- Provides framework for future enhancements

# 8. GUI interface for live simulation